PenPower A6 Scan Control SDK

.NET API Reference

Version 1.0.1

PPScanControl Namespace

Classes

ScanControl Class

Methods:

GetApiVersion

GetConnectedScanner

GetOpenedScanner

Open

IsCalibrateNeeded

Calibrate

StartScan

Close

Events:

ScanCompleted

ButtonPressed

ScanCompletedEventArgs Class

ButtonEventArgs Class

Enums

ScannerType Enum

ImageType Enum

ReturnCode Enum

ScanControl Class

- GetApiVersion

Get the version of the SDK library.

string GetApiVersion();

Parameter: void

Return value: string

Returns the version number of the SDK library, in a three-number dotted format string (ex: “1.0.0”).

- GetConnetedScanner

Get the current connected scanner device ID.

ScannerType GetConnectedScanner();

Parameter: void

Return value: ScannerType

Returns which type of scanner hardware is detected on the USB bus. If more than one type of scanners is connected, ID A6_7P3 would be returned.

- GetOpenedScanner

Get the currently opened scanner ID.

ScannerType GetOpenedScanner();

Parameter: void

Return value: ScannerType

Returns which scanner is currently opened by scan control SDK.

- Open

Open the specified scanner device.

ReturnCode Open(ScannerType scannerType);

Parameters:

ScannerType scannerType

[In] Specify the ID of the device to initialize.

Return value: ReturnCode

See return code definition.

- IsCalibrateNeeded

Test if the currently opened scanner needs calibration.

ReturnCode IsCalibrateNeeded();

Parameters: void

Return value: ReturnCode

If the scanner needs calibration, this method returns NeedCalibrate, If the scanner doesn’t need calibration, this method returns Success. For other errors, see return code definition.

- Calibrate

Calibrates the currently opened scanner.

ReturnCode Calibrate();

Parameter: void

Return value: ReturnCode

Returns Success if the process ended normally, or NeedCalibrate if failed. For other errors, see return code definition.

Remarks:

This method blocks until calibrate process returns. This method does not have any calibrating UI.

- StartScan

Start a scan procedure.

ReturnCode StartScan(int dpi, ImageType type, string outputFileName);

Parameter:

int dpi

[In] Specify the scanning DPI. Scanner ID A6_73F has minimum value 300, and A6_7P3 has minimum value 200.

ImageType type

[In] Specify the output image file’s format.

string outputFileName

[In] Specify the output image file’s full path. If this file already exists, it will be overwritten. If outputFileName is null, the SDK returns the result image in-memory instead of on-disk.

Return value: ReturnCode

See error code definition.

Remarks:

This method returns immediately. If this method succeeds, result of the scanning procedure would be raised through ScanCompleted event.

- Close

Close any opened scanner, abort scanning procedure. All the resources would be released, and the connected device would be uninitialized.

void Close();

Parameter: void

Return value: void

- ScanCompleted Event

Event for a scanning procedure completed.

event EventHandler<ScanCompletedEventArgs> ScanCompleted;

Remarks:

This event is raised after an image is scanned, or an error occurred during scanning/image processing. See definition of ScanCompletedEventArgs class.

- ButtonPressed Event

Event for a hardware button press.

event EventHandler<ButtonEventArgs> ButtonPressed;

Remarks:

This event is raised after a button on A6_7P3 scanner is pressed, other scanners will not raise this event. See definition of ButtonEventArgs class.

ScanCompletedEventArgs Class

Fields

ReturnCode Result

See error code definition.

string FileName

The output image file name.

If Result is Success and outputFileName is specified when start scanning, this field contains the same full file path as outputFileName of the corresponding StartScan method. Otherwise, this filed is null.

string ImageData

The output in-memory image data, as a base-64 encoded string.

If Result is Success and outputFileName is null when start scanning, this field contains the image data. Otherwise, this filed is null.

ButtonEventArgs Class

Fields

int ButtonId

The identifier of pressed buttons on A6_7P3 scanner. [Scan] button is 1, and [Custom] button is 2.

Return code definition

public enum ReturnCode

{

Success = 0,

Busy,

InvalidParameter,

IOException,

ScannerNotFound,

ScannerOpenFailed,

NoPaper,

PaperJam,

NeedCalibrate,

DllNotFound,

DriverError,

Uninitalized,

OtherError

}

| Error code | Value | Explanation |
| --- | --- | --- |
| Success | 0 | Operation succeeds. |
| Busy | 1 | Another operation is occupying the SDK. |
| InvalidParameter | 2 | Given scanning parameter is invalid. |
| IOException | 3 | IO errors occurred during scanning procedure. |
| ScannerNotFound | 4 | Given scanner ID is not connected or invalid. |
| ScannerOpenFailed | 5 | Cannot open the scanner. |
| NoPaper | 6 | The scanner has no paper inside feeder. |
| PaperJam | 7 | The scanner has paper jammed inside feeder. |
| NeedCalibrate | 8 | The scanner needs calibrate. |
| DllNotFound | 9 | An underlying assembly cannot be loaded. |
| DriverError | 10 | An error detected in the scanner driver. |
| Uninitalized | 11 | This method required Open being called first. |
| OtherError | 12 | Other uncategorized errors. |

ScannerType definition

Specify the scanner ID. A6_7P3 is the 2022 new model (black color). A6_73F is the old model (white color).

public enum ScannerType

{

None = 0,

A6_7P3,

A6_73F

}

ImageType definition

public enum ImageType

{

BMP = 0,

JPG,

PNG

}